Instruction for Building BIOS2200 CP/M 2.2 System

Date 22-Jun-83

���������1) Make any corrects/additions to the distribution copy of

���������   BIOS220� t� suppor� you� computer� seria� I/� hardware

���������2) Use MOVCPM.COM (supplied with the distribution disk of

���������   your CP/M v2.2 system) to generate a 60k CP/M system.

���������   Use the following command:

A>MOVCPM 60 *

A>SAVE 34 CPM60.COM

���������3) Using M80.COM (or a compatible assemblier), assemble

���������   BIOS2200.MAC into BIOS2200.REL.  Verify that no errors

���������   occurred during this process.  Use the following

���������   command:

A>M80 =BIOS2200

���������4) Now the BIOS2200.REL file must be converted into a Intel

���������   hex-ascii format (.HEX) file before it can be placed in

���������   th� CPM60.CO� file�  T� d� this� L80.CO� (Micro-Sof�'s

���������   linker supplied with M80.COM) must be used.  The command

���������   line looks like this:

A>L80 BIOS2200/N,BIOS2200/P:EA00/X/E

���������   After the link, make sure that the end of the BIOS module

���������   dosn't wrap around back to zero (BIOS too big). Answer

���������   NO to the question about moving the module above loader

���������   memory.

���������5) Use DDT.COM to perform the system patching as follows.

���������   A) First load DDT and CPM60.COM with the following

���������      command:   A>DDT CPM60.COM

���������   B) Next include the BIOS module with the correct offset

���������      to load it into the right place in the system, use

���������      the commands:

���������

-IBIOS2200.HEX<cr>

-R3580<cr>

���������      1] It should be noted at this point that no cold start

���������         booter is placed on the system tracks of the disk.

���������         The sector which would normally be used for this

���������         purpose is free for use by the user for any purpose

���������         desired.  The cold start booter is assumed to be in

���������         ROM.

���������   C) Make note the the NEXT field which is printed by DDT

���������      After loading the BIOS module.  Take the two most

���������      significant digits of the next field and convert this

���������      value to decimal.  Add one to this value and note this

���������      value for step 5E below.

���������   D) Type ^C (control C) to exit to the operating system.

���������   E) Save the newly created system with the SAVE command.

���������      Compute the number of pages to save from the NEXT field

���������      note� i� ste� 5� above�  Nam� th� save� fil� CPM60.SYS.

���������6) Place a cartriage which will receive the system into the

���������   first IOMEGA drive.

���������7) Run the supplied program IPLGEN00.

���������   A) Type 'M' (for make system) to the first prompt.

���������   B) Type the name of the file containing the gen'ed

���������      system to the next file, this is CPM60.SYS.

���������   C) Type the Correct Drive to the drive selection prompt.

���������   D) Following this you will have a IOMEGA cartrige with

���������      a system track on it.

#########################################################################################################