INDEX TO CORMS24                                                 2-15-85

1. INDEX.DOC

This index file.

2. CONFIG.SYS

An MSDOS (version 2/3) system configuration file. This file

must be copied over to your MSDOS boot disk.

3. CORDRV.BIN

The Corvus-IBM (PC, XT, AT) MSDOS (version 2/3) attachable

disk driver. This driver provides the system dependent resources

needed by MSDOS as well as Corvus MSDOS programs to use the Corvus

network and disk drives.

NOTE:   DO NOT use the DISKCOPY, SYS, or FORMAT programs with a

Corvus volume as the result is unpredictable and data

could be destroyed. To copy files when Corvus volumes are

involved use the COPY command or some other file oriented

copy program.

4. SPLDRV.BIN

The Corvus-IBM (PC, XT, AT) MSDOS (version 2/3) attachable printer

driver. This driver allows programs to communicate to a network

printer in the same manner as a local printer connected directly to

the machine.

5. PBOOT.DAT

A data file used by CORDRV.BIN when logging into the disk management

utilities (that run under the UCSD p-system).

6. CSD.COM

Utility that changes the parameters of SPLDRV.BIN.

7. SEMA4.PAS and SEMA4.EXE

An example program written with the Microsoft-IBM Pascal compiler.

It shows how to use the SEMAPHORE facility provided by the Corvus

drives. Access to SEMA4.PAS should be restricted as indiscriminate

use could confuse applications/utilities using semaphores.

8. DRIVEC2.DOC

Document file describing the support services provided by the Corvus

drive interface unit, DRIVEC2.OBJ.

9. DRIVEC2.OBJ

Module that can be linked to an application providing an interface

to the Corvus drive.

10. MAKEDISK.BAT

MSDOS batch file that copies files needed to make a Corvus boot disk.