DEBUG.DOC

This disk (SBK #5) contains software which can be used to

debug the FlexOS-386 system.  The debugging configuration

used by DRI engineering consists of the following items:

1.  COMPAQ 386 with at least 2.0 Mb of memory.

2.  IBM-compatible serial adapter (COM 0).

3.  Serial terminal.

4.  A copy of SBK #5 (Debug Boot Disk, DOS high-capacity).

5.  OS image file (FLEX386.CMD) and symbols

(FLEX386.SYM) on SBK #5.

SASID386 ("Stand-Alone Symbolic Instruction Debugger") is

written to communicate  directly to COM-0 hardware.   Use

DISKCOPY  (see FlexOS User's Guide) to make a copy of SBK

#5. Do not use the original Debug Disk for debugging. Use

it as a backup.

The FlexOS-386  loadable  image  may be constructed  in 2

forms:  a DEBUG system (which is used with the Debug Disk

and SASID386), and a BOOTABLE system. The main difference

between  the two is the LINK 86 command  options  used to

link  the  systems.  The  Debug  Disk  contains  a  small

single-threaded OS which is loaded automatically when the

Debug Disk is booted.

To create a DEBUG system, the FlexOS image must be linked

to avoid interference with SASID386 or the debug OS.  The

debug  OS resides  from  paragraph  address  0 to 0x2000,

while SASID386 resides in paragraph  addresses  0x7d00 to

0xA000.   This leaves the area between  0x2000 and 0x7d00

paragraphs    to   load   the   FlexOS-386   load   image

(FLEX386.CMD).

There  are two parts to the FlexOS 386  load image - code

and data.   The size of each will vary depending  on what

modules are included  in the system.  Optional OS modules

include linkable  hard disk drivers,  DOS front-end,  and

Graphics  Interface.   To obtain the size of the code and

data in your image  file,  link the file once and LINK 86

will  display  the size of each image  part in hex bytes.

You can then use these numbers to determine where to load

the code and data.  A standard  DRI load address  for the

code section is paragraph 0x200c.  Having selected a code

load  address,  add the size of the code  to it (the code

size  displayed  by LINK  86 is in bytes)  to get  to see

where the code section ends.  Use that paragraph  address

to select the data absolute  load address.   For example,

suppose that the code size is 0x31036 bytes. Convert this

to 0x3104 paragraphs  and add it to the code load address

to get 0x5110. This is the 1st legal address for the data

part of the system. For safety we add 0x400 paragraphs to

this number. We now have 0x5510 as the data load address.

If your  system  is too big,  you will  be unable  to use

SASID386.   To save memory space, certain  drivers may be

dynamically  loaded instead of statically linked into the

FlexOS  image.   This will reduce  the size of FlexOS 386

load image.  (see FlexOS Configuration Guide for informa-

tion about loadable drivers).

The  link  input   file  (see  the  FlexOS   Programmers'

Utilities Guide for more information on LINK 86) would be

as follows:

FLEX386.CMD [code[abs[200c]],

data[abs[5510],origin[100],max[0]]] =

followed by the names of the objects or librarys that are

used to generate your system.

After  linking  the system,  copy  both  FLEX386.CMD  and

FLEX386.SYM to the root of your copy of the debug disk.

Now place the debug disk in the A:  drive and reboot your

machine.   A batch  file will be automatically  run which

loads SASID386.   When you see SASID386's  prompt on your

serial terminal,  you may type '?' for a list of commands

supported.

HELPFUL TIPS.

If you are adding  code  to the system,  but not changing

the data size, then place the data as high as possible so

that as the code size increases you can use the same link

input file.

Use  the patch  area  in the system  (symbol  patch)  for

simple conditional  passpoint(s)  routines  while running

SASID386.  See  the  file  SIDTOOLS.A86  for  details  on

writting more complex and useful conditional passpoints.

WARNING.

Setting conditional passpoints or using the trace intruc-

tions  while in certain "real time areas"  of the  system

can cause a General Protection error.