.... Notes on BIOS/DDINIT/SYSGEN versions released 26-August-82 ...

BIOS:

This BIOS release should represent what we will be calling a

'distribution' version of the source code for the BIOS. It has been

cleaned up so that most mere mortals should be able to assemble it and

produce a working system without having to do any patches or funny

stuff. The main module 'AUTOBIOS.MAC' has includes for all the other

files such as the boot loader, signon message, deblock routines, hard

disk driver and system variables EQU's. The following items are the

main ones people will be interested in;

MSIZE .... memory size in Kbytes. use 60k for floppy-only systems

and 59k for hard disk versions.

USRDRV ... user number (high nybble) and drive number (low nybble)

to log-on after cold start. use 00h unless you are strange.

SERIAL ... set to TRUE to cause CONIN to use serial input port and

CONOUT to use built-in crt. Same trick as Bigboard PFM

rom's 'S' command, but gets done automatically by boot.

WINCH .... set to 1 to enable XEBEC drivers to be assembled in with

bios. Resuting code is > 1500 bytes long, so requires 59k

memsize to run.

NFLOPPY .. number of floppys in system. set to 1 or 2 for hard disk

systems or you will not have enough memory for the disk

allocation vectors in most cases.

DOUBLE ... set to ture to enable double sided floppies. allocates

blocks on other side of disk and doubles directory size

DIRSIZ ... number of directory entries allocated. we may want to change

this for compatibility with other systems.

BLS ...... CP/M block size parameter (expressed in Kbytes). this should

probably not be changed from the 2K value now being used,

but we have it just in case.

FIVE

EIGHT .... floppy disk type being used.

LSTDAT

LSTCTL

LSTBAUD .. sio ports and ctc chanel for serial printer driver. set to

80h/81h/89h for side A or 82h/83h/88h for side B. The one

sector loader contains a short table of constants for sio

initialization of write reg #4,#3,#5 and the ctc. Edit this

table to change parity, baudrate etc. We may want to make

a fancier way to define these parameters, but this will do

for a while.

XONXOFF .. set to TRUE to enable this protocol in printer driver.

TXMASK ... set to bits to be tested for printer-ready in list routine.

possible bits are CTS/SYNC/DCD/TBE. (always test TBE, others

are optional depending on printer)

A new module called 'MEMDEF' has been added to the CBIOS. This

is simply a copy of the monitor rom variables declaration file with

the variables defined using EQU's instead of DEFS's. The main use of

MEMDEF is to allow us make symbolic references to the system vars in

the CBIOS cold start initialization. Use this trick to do things like

changing the disk step rate, motor turn-off delay, keyboard fifo size,

disk spin-up delay and goodies like that.

SYSGEN:

Type the following magic incantations after you have edited

your desired changes in the bios or the equates shown above.

A>M80 =AUTOBIOS

A>L80

*/P:E900          {if 59k MSIZE then use E500 here}

*AUTOBIOS

*AUTOBIOS/X/N/E

#ZSID CPM.COM     {assumes you created file by saying 'SAVE 34 CPM.COM'

after running MOVCPM or standard version of SYSGEN}

#M980,1F7F,4000

#F100,3FFF,0

#IAUTOBIOS.HEX

#R2000            {if 59k MSIZE then use 2400 here}

#M4000,55FF,1600  {M4000,55ff,1200 if 5 1/4 inch }

You now have a picture of the reserved tracks as required by

our SYSGEN parked in memory from locations 900h to 2BFFh. You can save

this on a file if desired, and then run SYSGEN (our version) to put

the system on the reserved tracks of a formatted DD disk. The memory

layout for the system tracks in SYSGEN8 is as follows;

address      sector

------       ------

900-97f ... t0/s1           one sector loader for CP/M cold boot

980-9ff ... t0/s2           signon message

A00-15FF ... t0/s3 to 26     BIOS code (1.5 to 2.5 Kbytes)

1600-2BFF ... t1/s1 to nn     CCP and BDOS portion of CP/M (5.5 Kbytes)

44 sectors if 128 bytes/sector

22 "        " 256 "          "

11 "        " 512 "          "

6 "       " 1024 "          "

Next info added by L.Koopman, to be compatible with the

Sysgen5 program as submitted by Caltec.

The memory layout for SYSGEN5 is as follows;

address      sector

------       ------

900-97f ... t0/s1           one sector loader for CP/M cold boot

980-9ff ... t0/s2           signon message

A00-11FF ... t0/s3 to 18     BIOS code (1.5 to 2 Kbytes)

1200-27FF ... t1/s1 to nn     CCP and BDOS portion of CP/M (5.5 Kbytes)

44 sectors if 128 bytes/sector

22 "        " 256 "          "

11 "        " 512 "          "

6 "       " 1024 "          "

##################################################################################################################